SAMPLE QUESTIONS FROM MY UPCOMING BOOK

Question 1

Which one of the following actions does not amount to an offence under the Tourism Authority Act 2006?

A permitting 14 persons to take a trip at sea on a pleasure craft bearing registration mark PC 2145 OL 12

B operating a pleasure craft at night alone

C failing to produce skipper’s licence or a copy thereof when requested by an authorised officer

D operating a pleasure craft at night without navigational lights

Question 2

The category C skipper’s licence (previously category A :navigation up to 12 nautical miles) enables the holder to operate

A  an INSIDE LAGOON (IL) class pleasure craft up to 12 nautical miles from the shore

B  an INSIDE LAGOON (IL) class pleasure craft up to only 200 metres from the shore

C an OUTSIDE LAGOON (OL) class pleasure craft up to 12 nautical miles from the shore

D an OUTSIDE LAGOON (OL) class pleasure craft anywhere without any restriction on the distance from the shore

Question 3

Imagine the starboard side of a craft is receiving wind. We therefore refer to this side as the

A windward side

B  starboard windward side

C  leeward side

D  starboard leeward side

Question 4

Consider your craft is adrift at night due to an engine breakdown and the only landmark visible to you is this lighthouse. You have a portable magnetic compass and you read its bearing as 270◦ C.

Your craft is therefore

A  west of the lighthouse

B north – east of the lighthouse

C south-east of the lighthouse

D east of the lighthouse

Question 5

If a craft is covering a distance of 4 nautical miles in 20 minutes, its speed shall be

A20 knots

B12 knots

C4 knots

D16 knots

Question 6

Your GPS is showing the position of your craft as 19◦ 53'.823 S  057◦ 55'.209 E. It therefore means that your craft is

Ain the Northern hemisphere and East of the Greenwich meridian

Bin the Southern hemisphere and West of the Greenwich meridian

Con a  latitude which is 19 degrees 53.823 minutes south of the equator

Don a longitude which is 57degrees 55.209 minutes west of the Greenwich meridian

Question 7

You are navigating your craft on a northerly course of 010◦ C and you find this navigational   buoy right ahead of you at a distance of less than 200 metres.

In order to stay in safe waters, you should

A alter course to port in order to leave the buoy on your starboard side

Balter course to starboard in order to leave the buoy on your port side

Cpass as close as possible to the buoy

Davoid altering course to starboard

Question  8

Imagine yourself to be the skipper of a craft which is about to enter a harbour from high seas and at the entrance you find only this buoy:

In this case, you should

A leave the buoy on your starboard side

B leave the buoy on your port side

C stop your craft because this buoy is indicating an area of danger

D stop and seek instructions from the Port Authority

Question 9

In Mauritius, we experience tides over an average period of 24 hours as follows:

A2 consecutive High Waters followed by 1 Low Water

B  1 High Water followed by 1 Low Water

C2 consecutive High Waters followed by 2 consecutive Low Waters

D2 High Waters and 2 Low Waters in the order HIGH – LOW –HIGH - LOW or LOW-HIGH – LOW – HIGH

Question 10

Under which of these circumstances shall a skipper be right in his decision not to comply with the International Regulations for the Prevention of Collisions at Sea (IRPCS)?

A when he is navigating in a channel and there is no other craft in the channel

B when there is a situation of danger which makes it necessary to manoeuvre differently for the safety and security of the craft and the persons onboard

C when he needs to reach his destination in time

D when navigating in an area where visibility is restricted

Question 11

What is meant by ‘maintaining a lookout at sea’?

A keeping watch by eyes only

B watching for presence of other ships on the radar only

C maintaining a listening watch on VHF channel 16 only

D using all available means available onboard the vessel to ascertain if there is any risk of collision

Question 12

A ‘safe speed’ is

Athe recommended highest speed for the number and capacity of engine/s fitted on a craft

Ba speed adopted by the skipper which shall enable him to take proper and effective action to prevent a collision

Cany speed provided it is less than 10 knots

Da speed which is determined by the boat manufacturer

Question 13

One of the factors that need to be taken into account when adopting a ‘safe speed’ is

A the prevailing visibility

B the age of the skipper

C the engine capacity

D the distance of the craft from the shore

Question 14

An ‘overtaken vessel’ is

A a power-driven vessel

B a sailing vessel which is also being propelled by machinery

C a vessel which is being approached from behind by another vessel

D a vessel which is approaching any other vessel

Question 15

Which one of the following is the recommended angle of approach when coming alongside a jetty for securing your pleasure craft?

A   5 degrees

B  60 degrees

C 90 degrees

D 30 degrees

Question 16

Which one of these observations at sea confirms, beyond doubt, that your craft is approaching a tropical storm?

A Heavy rainfall

B A drop in temperature

C An increase in wind speed

D A drop in atmospheric pressure of more than 3 mb after correcting for height of barometer above sea level

Question 17

Skippers of commercial pleasure craft have a legal obligation to conduct a safety briefing with all the persons onboard prior to casting off their pleasure craft for a trip. What is a safety briefing?

A A description of the places which the passengers will be visiting during the trip

B A brief on the security measures to be observed and actions to be taken in a ny emergency

C A brief on the experience of the skipper in skippering vessels at sea

D A description of the profile of the company for which they work

Question 18

Consider yourself to be operating a fully equipped power-driven pleasure craft at sea at 17 knots and all of a sudden, due to heavy rainfall, the visibility reduces drastically. Which one of the following is the first action to be taken?

A Call all vessels in the vicinity on VHF channel 16 and request their GPS positions

B Reduce speed

C Check the depth of water using the echo sounder

D Stop maintaining visual lookout

Question 19

Consider yourself to be operating a 12 metres long power-driven vessel inside Port Louis harbour during the day to proceed to Caudan jetty. Which one of the following actions is strictly forbidden inside the channel?

A using your VHF radio set on channel 16 without the permission of the Mauritius Port Authority

B using sound signalling apparatus such as a horn without the permission of the Mauritius Port Authority

C crossing the channel from one side to the other in such a way to impede the passage of a large vessel

D letting go the anchor in an emergency

Question 20

In a situation of “Man Overboard”, which one of these sequences is the recommended one to ensure that the best actions are being taken and in an order of priority?

AMayday call on VHF channel 16, press MOB switch on GPS, boat manoeuvre

BBoat manoeuvre, press MOB switch on GPS, throw lifebuoy towards the person

CThrow lifebuoy towards the person, place a lookout, boat manoeuvre

DThrow lifebuoy towards the person, Mayday call on VHF channel 16, boat manoeuvre

Question 21

The Authority or the person who takes the decision to abandon a vessel in an emergency and evacuate the crew and passengers on liferaft/s is the

A National Coast Guard

B Tourism Authority

C skipper of the vessel

D owner of the vessel

Question 22

Consider yourself to be the skipper of a 6 metres long pleasure craft which has collided with an unlit floating structure at night outside the lagoon. The impact has been so great that water has started getting in and one of the other two persons onboard has fallen at sea but cannot be seen. He was wearing a life jacket. Your first reaction should be to

Acall the National Coast Guard for immediate assistance

Btry to bail out the water to keep the vessel afloat

Cswim out to look for the man overboard

Ddirect the person with you to swim out to look for the man overboard

Question 23

Consider yourself to be the skipper onboard a pleasure craft off Port Louis and suddenly you hear this transmission on VHF channel 16 “MAYDAY MAYDAY MAYDAY”. This means that what shall follow is

A a  message concerning the safety of navigation

B a message concerning the status of a navigational aid

C a message concerning a situation of imminent danger to life

D a message concerning a tanker which has just anchored outside the harbour

Question 24

Which one of the following statements about VHF channel 16 is FALSE?

A Channel 16 is an international calling channel on the VHF marine band

B Channel 16 is an international distress channel on the VHF marine band

C Channel 16 is not to be used in an area where military or paramilitary exercises are being conducted

D Channel 16 is not to be kept busy for a long time.

Question 25

When you are using a fire extinguisher to put out a fire onboard a craft at sea, you must ensure that with respect to wind direction, you position yourself on the

A windward side

B starboard side

C leeward side

D port side